Go back to EE-201 Page

Summary of the lecture notes on simple frequency selective circuits

EE-201 (Hadi Saadat)

Low pass Filters are used to pass low-frequency sine waves and attenuate high frequency sine waves. The cutoff frequency c is used to distinguish the passband (c) from the stopband (c ).  An elementary example of two passive lowpass filter is given below.

V0Vi=11+jωRC                          (1)                          V0Vi=11+jωLR|Vo||Vi|=11+ωRC212               (2)                         |Vo||Vi|=11+ωLR212θ=−tan−1ωRC                         (3)                         θ=−tan−1ωLRAt cutoff frequency Gain =12.  Substituting in (2)  result inωc=1RC                                   (4)                            ωc=RL          From (3), we see that the phase angle at cutoff frequency is −45∘The ratio VoVi is shown by H(jω), and is called the frequency response transfer function. The gain versus frequency, and the phase angle versus frequency known as the frequency response is as shown.

High pass Filters are used to stop low-frequency sine waves and pass the high frequency sine waves. The cutoff frequency c is used to distinguish the stopband (c) from the passband (c ).  An elementary example of two passive highpass filter is given below.

V0Vi=ωRCωRC−j1                           (5)                          V0Vi=ωLRωLR−j1|Vo||Vi|=ωRCωRC2+112                (6)                          |Vo||Vi|=ωLRωLR2+112θ=tan−11ωRC                            (7)                         θ=tan−1RωLAt cut off frequency Gain =12.  Substituting in (6) result inωc=1RC                                                               ωc=RLFrom (7), we see that the phase angle at cutoff frequency is  45∘

The gain versus frequency, and the phase angle versus frequency known as the frequency response is as shown.

Bandpass Filters

(a) The Parallel RLC Resonance

A circuit is in resonance when the voltage and current at the input terminals are in phase.

The circuit admittance isY=1R+j(ωC−1ω L). At resonance Y is purely conductive and ωC−1ω L=0, thus ωo=1LC. The circuit admittance is minimum or the circuit impedance at resonance, given byZ(ωo)=R, is maximum. Thus, the output voltage at resonance is maximum and is given by Vo(ωo)=RIi

The frequencies ω1 andω2 at which the output power drops to one half of its values at the resonant frequency are called the half-power frequencies. At these frequencies also known as cutoff frequencies or corner frequencies, the output voltage is |Vo(ωc|=0.707|Vo(ωo)|. This circuit which passes all the frequencies within a band of frequencies (ω1<ω<ω2) is called a bandpass filter. This range of frequency is known as the circuit bandwidth.

β=ω2−ω1

The half-power frequencies are obtained from

|Vo(ωc)|=Ii1R2+ωC−1ωL212=12RIi

Solving for ωwe obtain

ω1=−12RC+12RC2+1LC, and ω2=12RC+12RC2+1LC

From the above, we obtain,ω2−ω1=1RC, or the circuit bandwidth is

β=1RC

The cutoff frequencies can be written in terms of ω0andβ as follow:

ω1=−β2+β22+ωo2, and ω2=β2+β22+ωo2

This shows that ωois the geometric mean ofω1 andω2, i.e.,

ωo=ω1ω2

Notice that β is inversely proportional to R, i.e., smaller R results in a larger bandwidth. The resonant frequency (ωo=1LC) is a function of L and C. Therefore, by adjusting L and C a desired resonant frequency is obtained, whereas by adjusting R, the bandwidth and the height of the response curve is adjusted. The sharpness of the resonance is measured quantitatively by the quality factor Q.  This is defined as the ratio of the resonant frequency to the bandwidth.

Q=ωoβ

Substituting for β=1RCand ωo=1LCthe quality factor can be expressed as

Q=ωoRC=RωoL=RCL

At resonance IL and IC are given by

IL=VojωoL=−jVoRQ=−jQIi  and    IC=jωoCVo=jQRVo=jQIi

As it can be seen at resonance depending on the Q factor, IL and IC can be many times the supply current (current amplification).

(b) The Series RLC Resonance

A circuit is in resonance when the voltage and current at the input terminals are in phase.

The circuit impedance isZ=R+j(ωL−1ωC). At resonance Z is purely resistive and ωL−1ω C=0, thus ωo=1LC. The circuit impedance at resonance, given byZ(ωo)=R is minimum, and the current is maximum. Thus, the output voltage at resonance is maximum and is given by Vo(ωo)=RI(ωo)

The frequencies ω1 andω2 at which the output power drops to one half of its values at the resonant frequency are called the half-power frequencies. At these frequencies also known as cutoff frequencies or corner frequencies, the output voltage is |Vo(ωc)|=0.707|Vo(ωo)|. This circuit which passes all the frequencies within a band of frequencies (ω1<ω<ω2) is called a bandpass filter. This range of frequency is known as the circuit bandwidth.

β=ω2−ω1

The half-power frequencies are obtained from

|Vo(ωc)|=RViR2+ω L−1ω C212=12Vi

Solving for ωwe obtain

ω1=−R2L+R2L2+1LC, and ω2=R2L+R2L2+1LC

From the above, we obtain,ω2−ω1=RL, or the circuit bandwidth is

β=RL

The cutoff frequencies can be written in terms of ω0andβ as follow:

ω1=−β2+β22+ωo2, and ω2=β2+β22+ωo2

This shows that ωois the geometric mean ofω1 andω2, i.e.,

ωo=ω1ω2

Notice that β is proportional to R, i.e., larger R results in a larger bandwidth. The resonant frequency (ωo=1LC) is a function of L and C. Therefore, by adjusting L and C a desired resonant frequency is obtained, whereas by adjusting R, the bandwidth and the height of the response curve is adjusted. The sharpness of the resonance is measured quantitatively by the quality factor Q.  This is defined as the ratio of the resonant frequency to the bandwidth.

Q=ωoβ

Substituting for β=RLand ωo=1LCthe quality factor can be expressed as

Q=ωoLR=1ωoRC=1RLC

At resonance VL and VC are given by

VL=jωoLI(ω0)=jQVi

VC=−j1ωoCI(ω0)=−jQVi

As it can be seen at resonance depending on the Q factor, VL and VC can be many times the supply voltage (voltage amplification).

For a circuit with a very high quality factor Q, the corner frequencies may be approximated to ω1=ωo−β2, and ω1=ωo+β2

Bandreject Filter

A bandreject filter is designed to stop all frequencies within a band of frequencies (ω1<ω<ω2). In the series RLC circuit consider the output across the series combination L and C.

The voltage gain magnitude is

|Vo||Vi|=ωL−1ωCR2+ωL−1ωC212

At ω=0, the inductor behaves like a short circuit, and the capacitor behaves like an open circuit, I=0and Vo=Vi, and the voltage gain is unity.  At ω=∞, the inductor behaves like an open circuit and the capacitor behaves like a short circuit, I=0and again Vo=Vi, and the voltage gain is unity.  At resonance Z is purely resistive and ωL−1ω C=0, thus ωo=1LC.  Since the numerator of the voltage gain is zero, the gain drops to zero at ωo.

The cutoff frequencies, the bandwidth, and the quality factor are the same as the series RLC bandpass filter,β=RL,  ω1=−β2+β22+ωo2,  and ω2=β2+β22+ωo2